Project Conception of Numeric Systems

AES Decoder

ISMIN 2A

FENG Yuan

December 2018

- Introduction

AES is an algorithmic data encryption standard that uses symmetric keys to encode and decode data.

The process of encoding and decoding is composed of 11 rounds.

The original data is processed by passing AddRoundKey in the first round, following by nine times in a set of four functions: SubBytes, ShiftRows, MixColumns, AddRoundKey, then passed into the last round with only SubBytes, ShiftRows and AddRoundKey.

For decoding, The encrypted data is processed by passing AddRoundKey in the first round, following by nine times in four steps: InvSubbytes, InvShiftRows, InvMixColumns, AddRoundKey, then passed into the last round with only InvSubBytes, InvShiftRows and ADDRoundKey. Figure 1.1 shows the pseudo code for AES decryption algorithm.

Figure 1.1  pseudo code for AES decryption algorithm

A 16-byte data block (128 bit i.e.) forms the message to be inputted into the decoder. A secret key used in the initial round and then derived in the next rounds is a parameter of the AES algorithm. An additional function is required for the generation of the round keys. This function is called Key Expansion.

In this project, VHDL is used to develop a AES decoder. VHDL is a hardware description language used in electronic design to describe digital systems. It has higher efficiency compared to high-level programming languages like C. It is also a good chance to apply our knowledge and comprehension of VHDL and numeric systems.

- General Design

Figure 2.1 shows the inputs and outputs of InvAES entity. A 128 bits ciphered text Data_i is sent into the decoder. Clock_i permits all the components in InvAES work simultaneously. Start_i boots the process of decryption. Reset_i allows to reset the decoder. Aes_on_o=1 when a whole decryption cycle finishes. Data_o outputs a 128 bits deciphered text.

Figure 2.1  InvAES entity

- Component Description

  - S_Box.vhd

SBox is a table for substitution. Index X comes from the high 4 bits (7 to 4) of the input while index Y comes from the low 4 bits (3 to 0) of the input. For encoding and decoding, we use different substitution table. Figure 3.1.1 shows the substitution table for decoding.

Figure 3.1.1  substitution table for decoding.

In this figure, for example, the input of S_Box is “11101101” (“ed” in hexadecimal). So index X is “1110” (“e” in hexadecimal), index Y is “1101” (“d” in hexadecimal). Therefore we can locate the substitution “53” in hexadecimal and translate it into binary “01010011” for output.

Figure 3.1.2 shows the entity S_Box.

Figure 3.1.2  entity S_Box

S_Box_i is a 8 bits input.

S_Box_o is a 8 bits output.

In the test bench of S_Box.vhd, “ed”, “ee”, “ef” are sent to S_Box_i as input, then S_Box_o outputs the right substitution.

Figure 3.1.3  waves of S_Box_tb.vhd

3.3  InvSubBytes.vhd

InvSubBytes is the first function for round 0 to round 9.

This function replaces each element of the input state by using the substitution table of SBox. Figure 3.3.1 shows its principle.

Figure 3.3.1  principle of InvSubBytes

Figure 3.3.2 shows the entity InvSubBytes.

Figure 3.3.2  entity InvSubBytes.

The input and output is a 4*4 state block (type_state), each element of the block contains 2 hexadecimal digits.

In order to substitute 16 elements, the entity of InvSubBytes uses S_Box as a component, and calls it 16 times for substitution. Figure 3.3.3 shows internal structure of InvSubBytes.

Figure 3.3.3  internal structure of InvSubBytes

The VHDL code of port map S_Box_i=>InvSubBytes_i(0)(0), S_Box_o=>InvSubBytes_o(0)(0) indicates the substitution of one element by using S_Box, idem for other elements.

Figure 3.3.4 shows the wave of testbench simulation. Compared with substitution table for decoding, the output wave gives right result.

Figure 3.3.4 waves of InvSubBytes_tb.vhd

3.4  InvShiftRows.vhd

InvShiftRows is the second function of round 9 to round 0.

This function performs a cyclic permutation of the bytes of the state lines. The offset of the bytes corresponds to the index of the considered line (0 ≤ r <4).

For example: in line 3, x = {0xaa, 0xbb, 0xcc, 0xdd}, the application of the function will right shift it 3 times and get x’ = {0xbb, 0xcc, 0xdd, 0xaa}. In other words, S3,0S3,1S3,2S3,3 is replaced by S3,1S3,2S3,3S3,0.

An illustration of the functionality is detailed in Figure 3.4.1.

Figure 3.4.1  functionality of InvShiftRows

Figure 3.4.2 shows the entity InvShiftRows.

Figure 3.4.2  entity InvShiftRows

The input and output is a 4*4 state block (type_state), each element of the block contains 2 hexadecimal digits.

In VHDL code, InvShiftRows_o(3)(3)<=InvShiftRows_i(3)(0) indicates that S3,3 is replaced by S3,0 , idem for other elements in the state.

Figure 3.4.3 shows the wave of testbench simulation and gives the correct result.

Figure 3.4.3 waves of InvShiftRows_tb.vhd

3.5  KeyExpansion_table.vhd

This vhd file is provided. It’s functionality is to supply key for InvAddRound in each round.

Figure 3.5.1 shows the entity KeyExpansion_table.

Figure 3.5.1  entity KeyExpansion_table

The input round_i is bit4, which indicates the round.

The output gives a key of 128 bits according to the round.

3.6  InvAddRoundKey.vhd

InvAddRoundKey is the third function for round 0 to round 9.

This function adds the current round key (round 10 to round 0) to the state. It is equivalent to the AddRoundKey function in encoding process. Therefore, the Boolean XOR function is applied bit-by-bit between the bytes of the state and the bytes of the round key.

Figure 3.6.1 shows the calculation of InvAddRoundKey function.

Figure 3.6.1  calculation of InvAddRoundKey

Figure 3.6.2 shows the entity InvAddRoundKey.

Figure 3.6.2  entity InvAddRoundKey

The input InvAddRoundKey_i and output InvAddRoundKey_o is type_state.

The input InvAddRoundKey_round_i is 4 bits which indicates the round.

KeyExpansion_table is integrated as component in the InvAddRoundKey entity. Figure 3.6.3 shows internal structure of InvAddRoundKey.

Signal ek_s temporally saves the key of current round, it is divided into continuous bit8 as ek0_s ... ek15_s, then XOR with each element of input state.

VHDL code InvAddRoundKey_o(0)(0)<= ek0_s xor InvAddRoundKey_i(0)(0) represents one of the XOR operation, idem for other 15 elements.

Figure 3.6.3  internal structure of InvAddRoundKey

Figure 3.6.4 shows the waves of testbench simulation and gives the correct results of round 10 and round 7.

Figure 3.6.4 waves of InvAddRoundKey_tb.vhd

3.7  InvMixColumns.vhd

InvAddRoundKey is the fourth function for round 1 to round 9.

It applies a column-by-column transformation to a state. This linear transformation of a matrix product using the 4 bytes of a column. The columns are treated as polynomials in GF (28)2 and multiplied modulo x4 + 1 (noted ⊗) with the fixed polynomials in Figure 3.7.1. These operations performed in the Welsh field are calculated for the additions using an XOR function.

Figure 3.7.1  polynomials for InvMixColumns

Figure 3.7.2 shows the entity InvMixColumns.

Figure 3.7.2  entity InvMixColumns

The input InvMixColumns _i and output InvMixColumns _o is type_state.

Figure 3.7.3  internal structure of InvMixColumns

All the calculation of InvMixColumns is written in 3.8 Multiply.vhd. In order to calculate 4 columns, the entity of InvMixColumns uses Multiply as a component, and calls it 4 times for calculation of each column. Figure 3.7.3 shows internal structure of InvMixColumns.

Figure 3.7.4 shows the wave of testbench simulation and gives the correct result of round 7.

Figure 3.7.4 waves of InvMixColumns_tb.vhd

3.8  Multiply.vhd

Multiply is a component of InvMixColumns and it calculates one column of the input state by using invMixColumn algorithm. Figure 3.8.1 shows the entity Multiply.

Figure 3.8.1  entity Multiply

From Figure 3.7.1, we know that each element must have to be multiplied (modulo x4 + 1) by {0E},{0B},{0D} and{09}.

So take S0,0 as example:

{0E}S0,0 = (23+22+21) S0,0 = 8S0,0⊕4S0,0⊕2S0,0

{0B}S0,0 = (23+20) S0,0 = 8S0,0⊕S0,0

{0D}S0,0 = (23+22+20) S0,0 = 8S0,0⊕4S0,0⊕S0,0

{09}S0,0 = (23+21+20) S0,0 = 8S0,0⊕2S0,0⊕S0,0

That also means each element must have to be multiplied by 8, 4 and 2.

Suppose S0,0 = a7 a6 a5 a4 a3 a2 a1 a0 , so

{02}S0,0 = a6 a5 a4 a3 a2 a1 a0 0   when a7 = 0

{02}S0,0 = a6 a5 a4 a3 a2 a1 a0 0 ⊕ 00011011   when a7 = 1

In VHDL code, the calculation of {02}S0,0 is implemented as:

ER0_Mul2_s <= ( ElementRow0_s(6 downto 0) & '0' ) xor ( "000" & ElementRow0_s(7) & ElementRow0_s(7) & '0' & ElementRow0_s(7) & ElementRow0_s(7) );

which evades the judgement of a7.

Then it is easy to calculate {04}S0,0 , {08}S0,0 , {0E}S0,0 , {0B}S0,0 , {0D}S0,0 , {09}S0,0 .

Finally, use the formula in Figure 3.7.1 to calculate final result and send it to output.

3.9  InvAESRound.vhd

InvAESRound integrates four functions: InvSubbytes, InvShiftRows, InvMixColumns, InvAddRoundKey, which compose a whole round.

Figure 3.9.1 shows the entity InvAESRound. Calculation will be triggered by up edge of clock_i when resetb_i=1.

Figure 3.9.1  entity InvAESRound

Figure 3.9.2 shows internal process of InvAESRound. The internal functions (InvSubbytes, InvShiftRows, InvMixColumns, InvAddRoundKey) are used according to the values of enableInvMixcolumns_i and enableRoundcomputing_i.

There are internal signals to save current state or transform text between bit128 and type_state.

Figure 3.9.2  internal process of InvAESRound

Figure 3.9.3 shows the wave of testbench simulation and gives the correct result of round 10 and round 7.

Figure 3.9.3 waves of InvAESRound_tb.vhd

3.10  FSM_InvAES.vhd

FSM_InvAES is a finite-state machine (type Moore) , whose functionality is to generate the signals that controlling the entity InvAES.

Figure 3.10.1 shows the entity FSM_InvAES.

Figure 3.10.1  entity FSM_InvAES

Figure 3.10.2 shows the state transition diagram. There are totally 5 states (init, waiting, round10, round9to1, round0). Besides that, state_present saves present state while state_future saves future state.

The FSM boots when resetb_i=0. Present state enters future state when resetb_i=1 and is triggered by an up edge of clock_i.

When round_i=A, the state should enter state round9to1 in advance because state_present has one clock period lag than state_future, idem when round_i=1, round9to1 should enter round0.

Figure 3.10.2  state transition diagram

Figure 3.10.3 shows the truth table of FSM.

|  | init | waiting | round10 | round9to1 | round0 |
| --- | --- | --- | --- | --- | --- |
| done_o | 0 | 0 | 0 | 0 | 1 |
| enableCounter_o | 0 | 0 | 1 | 1 | 1 |
| enableMixcolumns_o | 1 | 1 | 0 | 1 | 0 |
| enableOutput_o | 0 | 1 | 1 | 1 | 1 |
| enableRoundcomputing_o | 1 | 1 | 1 | 1 | 0 |
| getciphertext_o | 0 | 1 | 1 | 0 | 0 |
| resetCounter_o | 0 | 0 | 1 | 1 | 1 |

Figure 3.10.3  truth table of FSM

Figure 3.9.3 shows the wave of testbench simulation. State_present enters next state correctly. The FSM works according to state transition diagram and truth table.

Figure 3.9.3  waves of FSM_InvAES_tb.vhd

3.11  Counter.vhd

The counter is controlled by FSM to fix the address of current round key.

Figure 3.11.1 shows the entity Counter.

Figure 3.11.1  entity Counter

Figure 3.9.3 shows the wave of testbench simulation, when enable_i=1 and resetb_i=1, the counter outputs correct signal for counting rounds.

Figure 3.9.3  waves of Counter_tb.vhd

3.12  RTL_MUX.vhd

RTL_MUX is a multiplexer to control the text which is sent to InvAESRound for decoding.

Figure 3.12.1 shows the entity RTL_LUX.

Figure 3.12.1  entity RTL_LUX

Input signal text0_i gets ciphered 128 bits original text.

Input signal text1_i gets deciphered 128 bits text from InvAESRound.

Input signal getText_i comes from getciphertext_o of FSM_InvAES.

If getText_i=1, multiplexer outputs text0_i.

If getText_i=0, multiplexer outputs text1_i.

Figure 3.12.2 shows the waves of RTL_MUX_tb.vhd.

getText_i=1, multiplexer outputs text0_i.

Figure 3.12.2  waves of RTL_MUX_tb.vhd.

3.13  InvAES.vhd

InvAES integrates all previous components (Counter, FSM_InvAES, InvAESRound, RTL_MUX). Figure 3.13.1 shows the global architecture of InvAES.

Figure 3.13.1  global architecture of InvAES

RTL_MUX2 is another multiplexer. If it receives enableOutput_o=1 from FSM, the multiplexer outputs decipher result from InvAESRound, else the multiplexer outputs 128 bits of zero.

RTL_Inv receives reset_i then outputs its inverse signal reset_s.

Figure 3.13.2 and Figure 3.13.3 show the waves of InvAES_tb.vhd. The AES Decoder receives ciphered text X“d6efa6dc4ce8efd2476b9546d76acdf0” then outputs the right deciphered result X“526573746f20656e2076696c6c65203f” after state round0.

Decode result of each round is correct.

Figure 3.13.2  waves of InvAES_tb.vhd

Figure 3.13.3  waves of InvAES_tb.vhd

- Conclusions

An in-depth comprehension of the AES decryption numeric system and a reasonable and effective application of VHDL is the basis for completing this project.

In the process of system designing, coding and debugging, I found and solved many problems and difficulties. Here lists some typical examples of them:

- If or when..else ?

In RTL_MUX (multiplexer), I used 2 if conditions in a process to control the output (text_o[127:0]):

if getText_i='1' then

text_s<=text0_i

end if;

if getText_i='0' then

text_s<=text1_i

end if;

When I did the simulation of InvAES_tb.vhd, I found currenttext_i do not update in the following rounds. So I deleted the process and used when..else to replace 2 if conditions:

text_s<=text0_i when getText_i='1' else

text1_i when getText_i='0';

then the problem solved.

The reason is that 2 if conditions are executed sequentially, in simulation the code was only executed once, that made currenttext_i do not update. But when..else is executed parallelly.

- Lag between state_present and state_future

There is a lag of one clock period between state_present and state_future, which will cause false calculation in simulation. The solution is let the state enter next state in advance of one round:

(In FSM_InvAES.vhd)

when round10=>

if round_i=X"a" then

state_future<=round9to1;

end if;

when round9to1=>

if round_i=X"1" then

state_future<=round0;

end if;

- How to evade the judgement of a7?

In Multiply.vhd, each element must have to be multiplied (modulo x4 + 1) by 2.

Suppose S0,0 = a7 a6 a5 a4 a3 a2 a1 a0 , so

{02}S0,0 = a6 a5 a4 a3 a2 a1 a0 0   when a7 = 0

{02}S0,0 = a6 a5 a4 a3 a2 a1 a0 0 ⊕ 00011011   when a7 = 1

The judgement of a7 will create many complicated condition sentences. In order to evade them, the calculation of {02}S0,0 can be implemented as:

ER0_Mul2_s <= ( ElementRow0_s(6 downto 0) & '0' ) xor ( "000" & ElementRow0_s(7) & ElementRow0_s(7) & '0' & ElementRow0_s(7) & ElementRow0_s(7) );

ElementRow0_s(6 downto 0) & '0' left shifts S0,0.

When a7=0, left shifted S0,0 XOR with “00000000”, the result is S0,0 itself.

When a7=0, left shifted S0,0 XOR with “00011011”.